### Nec2 Short reference card

| 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Geometry cards |  | I1 | I2 | F1 | F2 | F3 | F4 |  |  |  |  |  |
| Prog-ctrl cards |  | I1 | I2 | I3 | I4 | F1 | F2 | F3 | F4 | F5 | F6 |  |
| GW | Wire geom | Tag Nr | nr of segs | X1 | Y1 | Z1 | X2 | Y2 | Z2 | wire radius |  |  |
| GE | no gnd | 0 |  | no ground plane |  |  |  |  |  |  |  |  |
|  | ground plane | 1 |  | (ground plane present, mirror structure, current expansion; GN card required; screen- and wire radius on GN card should be blank) |  |  |  |  |  |  |  |  |
|  | ground | -1 |  | (ground present, mirror structure, no current expansion; GN card required; |  |  |  |  |  |  |  |  |
| EX | volt src | 0 | tag nr | segm nr | XX -> | real volts | Imag volts | (19: 0 No act.; 1 print rel.admit. matrix asymetry) (20: 0 No act.; 1 print imp’s for frequency loop) |  |  |  |  |
| FR | linear | 0 | nr of steps | 0 | 0 | start Mc | Step size |  |  | F1 = F0 + step |  |  |
|  | log | 1 | nr of steps | 0 | 0 | start Mc | Step size |  |  | F1 = F0 * step |  |  |
| GN | free space | -1 |  |  |  |  |  |  |  | Nullifies previous ground settings |  |  |
|  | finite ground | 0 | nr rad. wires | 0 | 0 | diel. Const | Cond. S/m | radius screen | radius wires | (In meters) See also GE card RP card must be set to 4 |  |  |
|  | perfect ground | 1 |  |  |  |  |  |  |  |  |  |  |
|  | sommer norton | 2 | 0 | 0 | 0 | diel. Const | Cond. S/m |  |  | note: conductivity must be negative for frequency loop |  |  |
| LD | nullify | -1 |  |  |  |  |  |  |  |  |  |  |
|  | serie RLC | 0 | tag nr | start segm | 0/end segm | R ohms | L Henry | C Farad |  | LD card always in series with EX and TL cards |  |  |
|  | parall | 1 | see 0 |  |  | see 0 |  |  |  |  |  |  |
|  | serie RLC | 2 | see 0 |  |  | R oh/m | L H/m | C F/m |  |  |  |  |
|  | parall | 3 | see 0 |  |  | see 2 |  |  |  |  |  |  |
|  | impe-dance | 4 | see 0 |  |  | Resis. Ohms | React. Ohms | - |  | No automatic frequency scaling |  |  |
|  | wire cond. | 5 | see 0 |  |  | Cond. mho/m |  |  |  |  |  |  |
| (*) | LC trap | 6 | See 0 |  |  | Q-coil | L henry | C farad |  |  |  |  |
| (*) | Insula-ted wire | 7 | See 0 |  |  | Diel. const | Coat radius |  |  | Grounds: cond: diel: Sea water: 5.0 80 Good ground: .01 10 Poor ground .001 4 Polar ice .0001 1 Fresh water .002 80 excellent .03 20 good .006 14 avarage .005 13 poor .002 12 |  |  |
| PQ | no chargs | -1 |  |  |  |  |  |  |  |  |  |  |
|  | chargs | 0 | tag nr | start segm | 0/end segm |  |  |  |  |  |  |  |
| PT | all curr | -2 |  |  |  |  |  |  |  |  |  |  |
|  | no curr | -1 |  |  |  |  |  |  |  |  |  |  |
|  | curr. | 0 | tag nr | start segm | 0/end segm |  |  |  |  |  |  |  |
|  | receiv pattrn | 1..3 | tag nr | start segm | 0/end segm |  |  |  |  | See page 74 manual |  |  |
| TL | trans line | tag-nr port 1 | seg-nr port1 | tag-nr port 2 | seg-nr port 2 | imped ohms | Length mtrs | admit real 1 | admit ima 1 | admit real 2 | admit ima 2 |  |
|  |  | a) Multiple ports are connected in parallel b) If connected to segment with LD; LD is in serie with TL |  |  |  |  |  |  |  |  |  |  |
| RP | normal | 0 | theta steps | phi steps | XNDA | theta start | Phi start | Theta stsize | phi stsize | far fld dist. | norm gain F |  |
|  | add surf wave | 1 | see 0 | XNDA: 17: 0: major/minor axis; 1: vert/hor gain 18: 0: no norm gain; 1-5: normalized gain 19: 0: power gain; 1: directive gain 20: 0: no avaraging; 1: avar gain; 2 avar gain (see also page 78 doc.) |  |  |  |  |  | Add surface wave |  |  |
|  | ground cond’s | 2..3 | see 0 |  |  |  |  |  |  | Special ground conditions |  |  |
|  | ground screen | 4 | see 0 |  |  |  |  |  |  | Ground-screen; must be specified in GN card ! |  |  |
|  | gnd scr, cond’s | 5..6 | see 0 |  |  |  |  |  |  | Botth ground-screen and special ground conditions |  |  |
| Geometry cards |  | I1 | I2 | F1 | F2 | F3 | F4 |  |  |  |  |  |
|  |  | 3-5 | 6-10 | 11-20 | 21-30 | 31-40 | 41-50 |  |  |  |  |  |
| Prog-ctrl cards |  | I1 | I2 | I3 | I4 | F1 | F2 | F3 | F4 | F5 | F6 |  |
|  |  | 3-5 | 6-10 | 11-15 | 16-20 | 21-30 | 31-40 | 41-50 | 51-60 | 61-70 | 71-80 |  |

Note:	- This summary is far from complete. It only lists the most important cards used by the author for his initial steps on the antenna-modeling path…

- Use the ‘Nec-editor’ (See ‘Settingssettings’ option on the ‘Main’ form) or consult the Nec-2 user-manual to assist with filling the appropriate positions for the different Nec-2 cards.