Description MW2Log.exe Version 1.70

MW2Log is a AddOn for MixW2 software. The tool is used for simple data input in case of „sent QSL“ and/or „rcvd QSL“. MW2Log save this information in the original logfile of MixW2.

Limits

MW2Log need one of the following country specific, system internal date format:

„DD.MM.YYYY“ (Europe)     „MM/DD/YYYY“ (USA)    „YYYY-MM-DD“ (ISO/JIS)

Version 1.70 can handle logfiles with up to 32'767 QSO.

Installation

The program does not need any installation. No system files are used or modified by MW2Log.exe.

Testing MW2Log (Important: Test it before use)

- Create a new directory

- Copy MW2Log.exe, MW2LogModes.txt and 170.ico into this directory

- Copy your original MixW2.log log file into this directory

- Create a new link to MW2Log.exe and use the icon 170.ico

Using MW2Log

- Copy MW2Log.exe in the directory of MixW2

- Copy MW2LogModes.txt and 170.ico into this directory

- Create a new link to MW2Log.exe and use the icon 170.ico

Logfile selection

This function was added to version 1.70. You are free to run MW2Log with all logfiles of MixW2.  Default logfile is MixW2.log. To select a other logfile please use the menu „Select Logfile“ and click to the needed entry in the list.

Display Log Data

- Direct Selection

All in the log existing calls are displayed in „List of Calls“. Standard scroll function is available. Several same calls show that there was more than only one QSO with the mentioned station is in the log. Select a call by clicking on the corresponding line. All QSOs with the selected call are displayed. For a new selection a further click into the list is needed. In case of more than 15 QSO’s an error message is given.

- Manual Selection

Select the field „Data Input“ and write the call you search. After each character all corresponding calls are displayed in the list „selected“. As longer your input is, as lower is the number of possible calls. In case of a complete call, QSO data are displayed, in case of more than 15 QSO’s an error message is given.

Selecting the flag „Autoselect“, more comfort is available. If only one call is in the list „selected“ all data are automatically displayed. For a new call input, please clear the field „Data Input“.

QSO selection

Setting the flags „open NO-QSL only“ only the contacts are displayed, which are not confirmed yet. Using the menu „Select Band“ and/or „Select Mode“ a further support is given to reduce the number of displayed QSOs. Using this function, handling of more than 15 QSOs per call is possible. Click into the red fields to reset the selection of band or mode.

Problem „Too much QSOs per call“

It is possible that more than 15 entries for one call are in the log, all not confirmed yet. In this case MW2Log.exe can not handle the data. Please process the data directly in MixW.

New Modes

Additional or new modes can be added. Use a simple editor to modify the file MW2LogModes.txt. Syntax must be the same as in MixW2. Do not delete the first line in this file.

„QSL sent“ and „New QSL received“

Set a flag in the column „New“ remarking the corresponding QSO. Setting more than one flag is possible. Before saving the data removing the flag is possible.

Save Data

Click the button „Save“ to save the data in MixW2.log. Removing new flags is not possible.

Clear all fields

Use button „Clear Screen“ to clear all fields except for „List of Calls“. This function can be used any time you want.

Backup MixW2.Log

MW2Log create a new directory „\MW2LogBackup“ for backup files. Every time you start the program, a backup of the original logfile is saved in the directory. The name of the backup file is „<name>_Logfile_<Date>_<Time>. bak“ . Use menu function „Data“ / „Delete Backup Logfiles“ to delete the backup files.

List of Backup Files

Use menu function „Data“ / „Show Backup Logfiles“ to see the list of backup files.

Current Version

Information is available in the head of the program.

Liability

„The author of MW2Log gives no guarantee for a flawless function of the product. He denies each liability for errors and damages, that emerge through the use of the software. The user takes the responsibility without reservation.“

Reference

Use menu function „Help“ > „About“ to read this information.

Exit application

Close the program using menu „File“ / „Exit“.

February 2004

HB9CIC Josef Rohner

Website www.qsl.net/hb9cic

Websites MixW2  www.mixw.net  /  www.mixw.de